# Recommended Equipment Stack for USA Battery Recycling Plant

## Hybrid Approach: Chinese Equipment + Western Safety Systems

Target Capacity: 1-2 t/h (1,000-2,000 kg/h) Estimated CAPEX: $500K-$1M (excluding installation) Payback Period: 24-30 months

## Equipment Recommendations

| Stage | Equipment | Supplier | Cost | Notes |
| --- | --- | --- | --- | --- |
| 1. Primary Shredder | Double-shaft shear, nitrogen-ready | Genox (China) or Xingmao (China) | $25K-45K | Verified USA installations; Genox has US office |
| 2. Hammer Mill | High-speed impact crusher, 1500 rpm | Xingmao or Henan Doing (China) | $15K-25K | 1500 rpm optimal per MDPI research |
| 3. Screening | Multi-deck vibrating screen (2mm, 5mm, 20mm) | Xingmao (integrated) | $8K-15K | Precision cut points critical for purity |
| 4. Magnetic Separator | Rare earth drum magnet | Eriez (USA) | $10K-20K | Western required - industry standard |
| 5. Eddy Current Separator | Non-ferrous metal separator | Eriez (USA) | $20K-35K | Higher recovery = faster payback |
| 6. Air Classifier | Density separation | Xingmao (integrated) | $10K-20K | Chinese adequate |
| 7. Dust Collection | HEPA filtration system | Xingmao + Western filters | $10K-18K | Use Western HEPA filters |
| 8. Nitrogen System | Membrane generator + O2 monitoring | Generon (USA) | $30K-50K | SAFETY CRITICAL |
| 9. Fire Suppression | Water mist + CO2 backup | Siemens (USA) | $25K-40K | SAFETY CRITICAL |
| 10. Gas Detection | HF, HCN, CO sensors | Drager or Honeywell (USA) | $15K-30K | SAFETY CRITICAL |

## Total Estimated Cost

| Category | Cost |
| --- | --- |
| Chinese Equipment (1-7) | $100K-180K |
| Western Safety + Separation (8-10) | $80K-135K |
| Equipment Total | $180K-315K |
| Installation, Integration, Commissioning | $80K-150K |
| Project Total | $300K-500K |

Note: Lower end = 500 kg/h, higher end = 1-2 t/h

## Why This Mix Works

- Chinese equipment (60-70% savings): Shredders, crushers, screens, and air classifiers are commodity equipment with mature global manufacturing

- Western safety (non-negotiable): Nitrogen systems, fire suppression, and gas detection require proven reliability for insurance approval

- Western separation (worth premium): Eriez separators deliver measurably better recovery rates - premium recovered in 12-18 months

## Top Supplier Contacts

| Supplier | Product | Website |
| --- | --- | --- |
| Genox (China) | Complete lines, shredders | genoxtech.com |
| Xingmao (China) | Complete lines, verified USA installs | lithiumbatterybroken.com |
| Eriez (USA) | Magnetic & eddy current separators | eriez.com |
| Generon (USA) | Nitrogen generators | generon.com |
| Drager (USA/Germany) | Gas detection systems | drager.com |
| Siemens (USA) | Fire suppression | siemens.com |

## Critical Success Factors

- Feedstock first - Secure 2-3 year battery supply agreements before purchasing

- Offtake contracts - Identify black mass buyers (Li-Cycle, Redwood, etc.)

- Safety budget - Do not reduce Western safety system allocation

- Installation planning - Budget 25-35% above equipment cost for installation

- Spare parts - Order 6-month spare parts inventory with equipment delivery